Entities & hardware:

“Entrance”:

- ESP 8266

- RFID – RC522 mini, black

  - Reads RF Chip

“Controller”:

- Server:

  - ESP 8266 Wi-Fi

  - Database storage (Storage\SD module)

“Rider”:

- ESP 8266 Wi-Fi

  - Communicate data with “Controller” (Receive & send)

  - Communicate data with Arduino 1 & 2 (Receive & send)

- Arduino Nano 1:

  - Ultrasonic HC-SR04 x2 (1 forward, 1 backward)

  - RFID – RC522 mini, black

    - Reads RF Sticker

  - Lithium charger – TP5000 (Battery status indication?)

  - DC driver MX1508

    - 4 Motors for “Rider” movement (Full movement Forward-backward)

- Arduino Nano 2:

  - Gyro MPU 6050

  - Stepper driver A4988 x4

    - Linear motors (‘Expend’-‘shrink’)

      - 2 Motors (360 degrees)

    - Rotational motors (Clockwise-Counterclockwise)

      - 2 Motors (180 degrees)

    - Gyro motor (Keep parallel to ground)

      - 2 Motors

    - Conveyor motor (Forward-backward)

      - 1  Motor

“Accessories”:

- RF Chip (Costumer)

- RF Sticker (Product)

“Pay-Box”:

- ESP 8266 Wi-Fi

  - Free = Can be occupied only by 1 costumer (RF Chip)

  - Occupied = If 1 costumer is assigned

- Micro-switch

  - Indicates “Rider” location at “Pay-Box”

Functions:

“Entrance”:

- Wait mode (RFID)

  - Wait to read an RF Chip

- Read RF Chip

  - Ask for Identification (Identity Card \ Credit card)

  - Send data to “Controller”

  - Assign a Costumer ID for the RF Chip

  - Assign a “Pay-Box” for the new ID

“Controller”:

- Receive data from all entities and their sensors (“Entrance”, “Rider”, “Accessories”, “Pay-Box”)

- Calculate data

- Save data and manage database

  - Product database

  - Costumer database

  - “Pay-Box” database

“Rider”:

- Wait mode

  - Wait to read RF Sticker

  - Waiting in ‘shrink’ status

- Movement wait mode (Wait for “Controller” to decide the route)

- Read Product RF Sticker (Receive weight and price from Product database)

- Accelerate and Decelerate motors (By calculating data)

- Stay parallel to the ground (Gyro)

- Send location status to “Controller” (Micro-Switch indicators)

- Send movement status to “Controller” (Current Speed, Acc\Dec, Stop)

- Sense objects in the way (Ultrasonic)

  - Stop “Rider” if an interference has been detected

- Conveyor belt exchange (Move product from “Rider” to another “Rider”)

  - Transfer product data

- Final destination (At the “Pay-Box”)

  - Update the “Controller” (By micro-switch)

  - Put product in place with the conveyor belt

  - Shrink (Linear motors)

  - Move back to base location (By commend from “Controller”)